A Big Earth Data Platform for Three Poles

Cold and Arid Research Network of Lanzhou university (an observation system of Meteorological elements gradient of Suganhu Station, 2020)

1、Description

This dataset includes data recorded by the Cold and Arid Research Network of Lanzhou university obtained from an observation system of Meteorological elements gradient of Suganhu Station from November 27 to December 31, 2020. The site (93.708° E, 40.348° N) was located on a wetland in the Suganhu west lake, Gansu Province. The elevation is 990 m. The installation heights and orientations of different sensors and measured quantities were as follows: air temperature and humidity profile (4m and 8 m, towards north), wind speed and direction profile (windsonic; 4m and 8 m, towards north), air pressure (1 m), rain gauge (4 m), infrared temperature sensors (4 m, towards south, vertically downward), soil heat flux (-0.05 and -0.1m ), soil soil temperature/ moisture/ electrical conductivity profile (below the vegetation in the south of tower, -0.05 and -0.2 m), photosynthetically active radiation (4 m, towards south), four-component radiometer (4 m, towards south), sunshine duration sensor(4 m, towards south).
The observations included the following: air temperature and humidity (Ta_4 m, Ta_8 m; RH_4 m, RH_8 m) (℃ and %, respectively), wind speed (Ws_4 m, Ws_8 m) (m/s), wind direction (WD_4 m, WD_8 m) (°), air pressure (press) (hpa), precipitation (rain) (mm), four-component radiation (DR, incoming shortwave radiation; UR, outgoing shortwave radiation; DLR_Cor, incoming longwave radiation; ULR_Cor, outgoing longwave radiation; Rn, net radiation) (W/m^2), infrared temperature (IRT) (℃), photosynthetically active radiation (PAR) (μmol/ (s m-2)), soil heat flux (Gs_0.05m, Gs_0.1m) (W/m^2), soil temperature (Ts_0.1m, Ts_0.2m, Ts_0.4m) (℃), soil moisture (Ms_0.1m, Ms_0.2m, Ms_0.4m) (%, volumetric water content), soil conductivity (Ec_0.1m, Ec_0.2m, Ec_0.4m)(μs/cm), sun time(h).
The data processing and quality control steps were as follows: (1) The AWS data were averaged over intervals of 10 min for a total of 144 records per day and missing records were denoted by -6999. (2) Data in duplicate records were rejected. (3) Unphysical data were rejected. (4) The data marked in red are problematic data. (5) The format of the date and time was unified, and the date and time were collected in the same column.

2、Keywords

Theme：Surface Water,Visibility
Discipline：Atmosphere,Terrestrial Surface
Places：Qaidam Basin, Sugan Lake
Time：2020

3、Data details

1.Scale：None

2.Projection：

3.Filesize：11.8MB

4.Data format：None

4、Space scope

| - | north：38.992 | - |
| --- | --- | --- |
| west：94.125 | - | east：94.125 |
| - | south：38.992 | - |

5、Time frame:2019-12-31 16:00:00+00:00--2020-12-30 16:00:00+00:00

6、Reference method

References to data:

ZHANG Renyi, ZHAO Changming. Cold and Arid Research Network of Lanzhou university (an observation system of Meteorological elements gradient of Suganhu Station, 2020). A Big Earth Data Platform for Three Poles, doi:10.11888/Meteoro.tpdc.2713792021

References to articles:

7、Supporting project information

Pan-Third Pole Environment Study for a Green Silk Road-A CAS Strategic Priority A Program

8、Data resource provider

name: ZHAO Changming
unit: Lanzhou University
email: zhaochm@lzu.edu.cn

name: ZHANG Renyi
unit: Lanzhou University
email: zrenyi@lzu.edu.cn