A Big Earth Data Platform for Three Poles

WATER: Dataset of ground truth measurements synchronizing with Landsat TM in the Linze grassland and  Linze station foci experimental area during the pre-observation period (on Sep. 23, 2007)

1、Description

The dataset of ground truth measurements synchronizing with Landsat TM was obtained in the Linze grassland and Linze station foci experimental area on Sep. 23, 2007 during the pre-observation periods, and one scene was captured well. These data can provide reliable ground data for retrieval and validation of land surface temperatures with EO-1 Hyperion remote sensing approaches.
    Observation items included:
    (1) the land surface radiative temperature by the hand-held infrared thermometer, which was calibrated; 
    (2) GPS by GARMIN GPS 76;
    (3) atmospheric parameters at Daman Water Management office measured by CE318 (produced by CIMEL in France). The total optical depth, aerosol optical depth, Rayleigh scattering coefficient, column water vapor in 936 nm, particle size spectrum and phase function were then retrieved from these observations. The optical depth in 1020nm, 936nm, 870nm, 670nm and 440nm were all acquired by CE318. These data include the raw data in .k7 format and can be opened by ASTPWin software. ReadMe.txt is attached for detail. Processed data (after retrieval of the raw data) in Excel contain optical depth, rayleigh scattering, aerosol optical depth, the horizontal visibility, the near surface air temperature, the solar azimuth, zenith, solar distance correlation factors, and air column mass number. 
    (4) ground-based land surface temperature measurements by the thermal imager in the Heihe gobi, west of Zhangye city.

2、Keywords

Theme：Surface radiation temperature,Earth SurFace Processes,Aerosol,Remote Sensing Technology,Aerosol optical depth/Thickness,Aerosol backscatter,Visible remote sensing,Solar spectrophotometer
Discipline：Atmosphere,Terrestrial Surface,Remote Sensing Technology
Places：Heihe River Basin, Arid Region Hydrology in the Middle Reaches, Zhangye City Foci Experimental Area, Closed observation area of Linze station, 
Time：2007,

3、Data details

1.Scale：None

2.Projection：4326

3.Filesize：99.4MB

4.Data format：EXCEL

4、Space scope

| - | north：39.379496 | - |
| --- | --- | --- |
| west：100.110909 | - | east：100.201591 |
| - | south：39.311923 | - |

5、Time frame:2007-10-06 00:00:00+00:00--2007-10-06 00:00:00+00:00

6、Reference method

References to data:

LI Hua, FANG   Li, BAI   Yunjie, PAN   Xiaoduo, WU   Yueru, YAN   Yeqing, DING   Songchuang, LI   Zhe, Liu  Qiang, GAO  Song, CHE   Tao, LIANG   Ji, Wen Jianguang, HAN   Xujun, ZHANG   Lingmei, YAN   Qiaodi, MA   Hongwei, WANG Xufeng, RAN  Youhua, LI Xin, YUAN   Xiaolong, LI   Hongyi, QIN   Chun, HAO Xiaohua. WATER: Dataset of ground truth measurements synchronizing with Landsat TM in the Linze grassland and  Linze station foci experimental area during the pre-observation period (on Sep. 23, 2007). A Big Earth Data Platform for Three Poles, doi:10.3972/water973.0193.db2013

References to articles:

7、Supporting project information

The CAS (Chinese Academy of Sciences) Action Plan for West Development Project
National Program on Key Basic Research Project (973 Program

8、Data resource provider

name: HAO Xiaohua
unit: Cold and Arid Regions Environmental and Engineering Research Institute, Chinese Academy of Sciences
email: haoxh@lzb.ac.cn

name: WANG Xufeng
unit: Cold and Arid Regions Environmental and Engineering Research Institute, CAS
email: wangxufeng@lzb.ac.cn

name: Wen Jianguang
unit: 
email: wenjg@irsa.ac.cn

name: LI Hua
unit: 
email: 

name: LI Xin
unit: 
email: xinli@itpcas.ac.cn

name: Liu  Qiang
unit: 
email: 

name: GAO  Song
unit: 
email: 

name: RAN  Youhua
unit: 
email: ranyh@lzb.ac.cn

name: QIN   Chun
unit: 
email: 

name: WU   Yueru
unit: 
email: 

name: LIANG   Ji
unit: 
email: leung@lzb.ac.cn

name: LI   Hongyi
unit: 
email: lihongyi@lzb.ac.cn

name: LI   Zhe
unit: 
email: 

name: DING   Songchuang
unit: 
email: 

name: FANG   Li
unit: 
email: li_fang113@163.com

name: MA   Hongwei
unit: 
email: 

name: YUAN   Xiaolong
unit: 
email: 

name: BAI   Yunjie
unit: 
email: baiyj27@163.com

name: YAN   Qiaodi
unit: 
email: 

name: ZHANG   Lingmei
unit: 
email: 

name: YAN   Yeqing
unit: 
email: 

name: CHE   Tao
unit: 
email: chetao@lzb.ac.cn

name: HAN   Xujun
unit: 
email: 

name: PAN   Xiaoduo
unit: Institute of Tibetan Plateau Research, CAS
email: panxd@itpcas.ac.cn