A Big Earth Data Platform for Three Poles

In-situ measurement data set (2018) of the soil moisture and temperature wireless sensor network within the Shandian River Basin

1、Description

This data set contains in-situ measurements of soil moisture, soil temperature and precipitation at 34 stations from a wireless Soil Moisture Network within the ShanDian River basin (referred to as the SMN-SDR hereafter). The coverage of entire network is about 10,000 km2 (115.5-116.5°E, 41.5-42.5°N). The topography of the SMN-SDR is relatively flat, and land surfaces are typically dominated by grasslands and croplands. A total of 34 stations are set up in the network with three sampling scales including 100 km (large scale), 50 km (medium scale), and 10 km (small scale). The soil moisture sensors used are Decagon 5TM with five measuring depths (3, 5, 10, 20, and 50 cm) installed for each station. Of the 34 station, there are 20 stations equipped with HOBO rain gauges. Undisturbed soil samples at each layer of soil for each station was taken to analyze the gravimetric/volumetric water content, bulk density, and soil texture for a further specific calibration. The power supply is provided by solar panels and all data can be transmitted wirelessly to a server. The sampling interval of the data recording time is 10 (before June 2019) or 15 minutes (after June 2019).
This network can improve the comprehensive observation capabilities of key water cycle parameters in the ShanDian River basin and provide long-term ground reference data for satellite- and model-based soil moisture products.

2、Keywords

Theme：Soil,Surface Water,Soil moisture,Soil moisture/Water content,Terrestrial Surface Remote Sensing,Ground verification information
Discipline：Terrestrial Surface
Places：the Luan River, the Shandian river basin
Time：2018 year

3、Data details

1.Scale：None

2.Projection：

3.Filesize：172.0MB

4.Data format：None

4、Space scope

| - | north：42.5 | - |
| --- | --- | --- |
| west：115.5 | - | east：116.5 |
| - | south：41.5 | - |

5、Time frame:2018-08-31 16:00:00+00:00--2018-12-31 03:59:59+00:00

6、Reference method

References to data:

HU   Lu, JI   Dabin, SHI   Jiancheng, ZHANG   Ziqian, ZHAO   Tianjie, CHEN   Deqinq, CUI   Qian, JIANG Lingmei, ZHENG   Jingyao. In-situ measurement data set (2018) of the soil moisture and temperature wireless sensor network within the Shandian River Basin. A Big Earth Data Platform for Three Poles, doi:10.11888/Soil.tpdc.2714252021

References to articles:

阎广建, 赵天杰, 穆西晗, 闻建光, 庞勇, 贾立, 张永光, 陈德清, 姚崇斌, 曹志宇, 雷永荟, 姬大彬, 陈良富,柳钦火, 吕利清, 陈镜明, 施建成. (2021). 滦河流域碳、水循环和能量平衡遥感综合试验总体设计. 遥感学报, 25(4), 856-870.

Zhao, T.J., Shi, J.C., Lv, L.Q., Xu, H.X., Chen, D.Q., Cui, Q., Jackson, T.J., Yan, G.J., Jia, L., Chen, L.F., Zhao, K., Zheng, X.M., Zhao, L.M., Zheng, C.L., Ji, D.B., Xiong, C., Wang, T.X., Li, R., Pan, J.M., Wen, J.G., Yu, C., Zheng, Y.M., Jiang, L.M.,
Chai, L.N., Lu, H., Yao, P.P., Ma, J.W., Lv, H.S., Wu, J.J., Zhao, W., Yang, N., Guo, P., Li, Y.X., Hu, L., Geng, D.Y., & Zhang, Z.Q. (2020). Soil moisture experiment in the Luan River supporting new satellite mission opportunities. Remote Sensing of Environment, 240. https://doi.org/10.1016/j.rse.2020.111680.

赵天杰, 施建成, 徐红新, 孙彦龙, 陈德清, 崔倩, 贾立, 黄硕, 牛升达, 李秀伟, 阎广建, 陈良富, 柳钦火, 赵凯, 郑兴明, 赵利民, 郑超磊, 姬大彬, 熊川, 王天星, 李睿, 潘金梅, 闻建光, 穆西晗, 余超, 郑姚闽, 蒋玲梅, 柴琳娜, 卢麾, 姚盼盼, 马建威, 吕海深, 武建军, 赵伟, 杨娜, 郭鹏, 李玉霞, 胡路, 耿德源, 张子谦,胡建峰, 杜爱萍. (2021). 闪电河流域水循环和能量平衡遥感综合试验. 遥感学报, 25(4), 871-887.

Zheng, J.Y., Zhao, T.J., Lü, H.S., Shi, J.C., Cosh, M.H., Ji, D.B., Jiang, L.M., Cui, Q., Lu, H., Yang, K., Wigneron, J.-P., Li, X.J., Zhu, Y.H., Hu, L., Peng, Z.Q., Wang, X.Y., & Kang, C.S. (2022). Assessment of 24 soil moisture datasets using a new in-situ network in the Shandian River Basin of China. Remote Sensing of Environment, 271. https://doi.org/10.1016/j.rse.2022.112891.

7、Supporting project information

Satellite observation and simulation studies of the land surface water and energy exchange processes and its effects on global changes

8、Data resource provider

name: JIANG Lingmei
unit: Beijing Normal University
email: jiang@bnu.edu.cn

name: JI   Dabin
unit: 
email: jidb@aircas.ac.cn

name: CUI   Qian
unit: 
email: cuiqian@mwr.gov.cn

name: ZHENG   Jingyao
unit: 
email: zhengjingyao94@gmail.com

name: ZHANG   Ziqian
unit: 
email: zhangzq55@mail2.sysu.edu.cn

name: SHI   Jiancheng
unit: 
email: shijiancheng@nssc.ac.cn

name: ZHAO   Tianjie
unit: 
email: zhaotj@aircas.ac.cn

name: CHEN   Deqinq
unit: 
email: chendq@mwr.gov.cn

name: HU   Lu
unit: 
email: hulu@smail.nju.edu.cn