A Big Earth Data Platform for Three Poles

Qilian Mountains integrated observatory network: cold and arid research network of Lanzhou university (an observation system of meteorological elements gradient of Xiyinghe station, 2018)

1、Description

This dataset includes data recorded by the Cold and Arid Research Network of Lanzhou university obtained from an observation system of Meteorological elements gradient of Xiyinghe Station from January 1 to December 31, 2018. The site (101.853E, 37.561N) was located on a alpine meadow in the Menyuan,Qinghai Province. The elevation is 3639 m. The installation heights and orientations of different sensors and measured quantities were as follows: air temperature and humidity profile (2, 4, and 8 m, towards north), wind speed and direction profile (windsonic; 2, 4, and 8 m, towards north), air pressure (1.5 m), rain gauge (4 m), four-component radiometer (4 m, towards south), infrared temperature sensors (4 m, towards south, vertically downward), photosynthetically active radiation (4 m, towards south), soil heat flux (-0.05 m and -0.1m in south of tower), soil soil temperature/ moisture/ electrical conductivity profile (-0.2 and -0.4 m in south of tower), sunshine duration sensor (4 m, towards south).
The observations included the following: air temperature and humidity (Ta_2 m, Ta_4 m, and Ta_8 m; RH_2 m, RH_4 m, and RH_8 m) (℃ and %, respectively), wind speed (Ws_2 m, Ws_4 m, and Ws_8 m) (m/s), wind direction (WD_2 m, WD_4 m, and WD_8 m) (°), air pressure (press) (hpa), precipitation (rain) (mm), four-component radiation (DR, incoming shortwave radiation; UR, outgoing shortwave radiation; DLR_Cor, incoming longwave radiation; ULR_Cor, outgoing longwave radiation; Rn, net radiation) (W/m^2), infrared temperature (IRT) (℃), photosynthetically active radiation (PAR) (μmol/ (s/m^2)), soil heat flux (Gs_5 cm, Gs_10cm) (W/m^2), soil temperature (Ts_20 cm, Ts_40 cm) (℃), soil moisture (Ms_20 cm, Ms_40 cm) (%, volumetric water content), soil water potential (SWP_20cm , SWP_40cm)(kpa) , soil conductivity (Ec_20cm, Ec_40cm)(μs/cm), sun time (h).
The data processing and quality control steps were as follows: (1) The AWS data were averaged over intervals of 10 min for a total of 144 records per day. The meteorological data were missing during Aug. 29 to Oct.18 because of unstable power supply due to battery box flooding; The wind speed and direction profile data were rejected because of sensor failure; The precipitation data were rejected because of program error; The air humidity data before Mar. 2 were rejected due to program error; (2) Data in duplicate records were rejected. (3) Unphysical data were rejected. (4) The data marked in red are problematic data. (5) The format of the date and time was unified, and the date and time were collected in the same column, for example, date and time: 2018-6-10 10:30.

2、Keywords

Theme：Soil,Precipitation,Desert,Vegetation,Soil moisture/Water content,Meteorological element
Discipline：Atmosphere,Terrestrial Surface
Places：Shiyang River Basin
Time：2018

3、Data details

1.Scale：None

2.Projection：None

3.Filesize：11.6MB

4.Data format：None

4、Space scope

| - | north：37.561 | - |
| --- | --- | --- |
| west：101.855 | - | east：101.855 |
| - | south：37.561 | - |

5、Time frame:2018-01-07 08:00:00+00:00--2019-01-06 08:00:00+00:00

6、Reference method

References to data:

ZHANG Renyi, ZHAO Changming. Qilian Mountains integrated observatory network: cold and arid research network of Lanzhou university (an observation system of meteorological elements gradient of Xiyinghe station, 2018). A Big Earth Data Platform for Three Poles, doi:10.11888/Geogra.tpdc.2701582019

References to articles:

7、Supporting project information

Pan-Third Pole Environment Study for a Green Silk Road-A CAS Strategic Priority A Program

8、Data resource provider

name: ZHAO Changming
unit: Lanzhou University
email: zhaochm@lzu.edu.cn

name: ZHANG Renyi
unit: Lanzhou University
email: zrenyi@lzu.edu.cn