A Big Earth Data Platform for Three Poles

HiWATER: Dataset of airborne microwave radiometers (L bands) mission in the river way of the midstream of the Heihe River Basin on Jul. 4, 2012

1、Description

The dataset of airborne Polarimetric L-band Multibeam Radiometers (PLMR) was acquired on 4 July, 2012, located along the riverway of Heihe River in the middle reaches of the Heihe River Basin. The aircraft took off at 10:50 am (UTC+8) from Zhangye airport and landed at 14:50 pm, with the flight time of 4 hours. The flight was performed in the altitude of about 1000 m and at the speed of about 220-250 km during the observation, corresponding to an expected ground resolution of about 300 m. 
The PLMR instrument flown on a small aircraft operates at 1.413 GHz (L-band), with both H- and V-polarizations at incidence angles of ±7.5°, ±21.5° and ±38.5°. PLMR ‘warm’ and ‘cold’ calibrations were performed before and after each flight. The processed PLMR data include 2 DAT files (v-pol and h-pol separately) and 1 KMZ file for each flying day. The DAT file contains all the TB values together with their corresponding beam ID, incidence angle, location, time stamp (in UTC) and other flight attitude information as per headings. The KMZ file shows the gridded 1-km TB values corrected to 38.5 degrees together with flight lines. Cautions should be taken when using these data, as the RFI contaminations are often higher than expected at v-polarization.

2、Keywords

Theme：Remote Sensing Technology,Airborne remote sensing,Microwave radiometer
Discipline：Remote Sensing Technology
Places：Heihe River Basin, the artificial oasis experimental area in the middle reaches, 
Time：2012, 2012-07-04

3、Data details

1.Scale：None

2.Projection：4326

3.Filesize：36.0MB

4.Data format：文本

4、Space scope

| - | north：39.5403 | - |
| --- | --- | --- |
| west：99.8517 | - | east：100.7906 |
| - | south：38.6272 | - |

5、Time frame:2018-11-22 10:50:03+00:00--2018-11-22 10:50:03+00:00

6、Reference method

References to data:

LI Xin. HiWATER: Dataset of airborne microwave radiometers (L bands) mission in the river way of the midstream of the Heihe River Basin on Jul. 4, 2012. A Big Earth Data Platform for Three Poles, doi:10.3972/hiwater.015.2013.db2017

References to articles:

Li, X., Liu, S.M., Xiao, Q., Ma, M.G., Jin, R., Che, T., Wang, W.Z., Hu, X.L., Xu, Z.W., Wen, J.G., Wang, L.X. (2017). A multiscale dataset for understanding complex eco-hydrological processes in a heterogeneous oasis system. Scientific Data, 4, 170083. doi:10.1038/sdata.2017.83.

7、Supporting project information

"Heihe Watershed Allied Telemetry Experimental Research (HiWATER)

8、Data resource provider

name: LI Xin
unit: 
email: xinli@itpcas.ac.cn