A Big Earth Data Platform for Three Poles

WATER: Dataset of LAS (large aperture scintillometer) observations at the A'rou freeze/thaw observation station (2008-2012)

1、Description

The dataset of LAS (Large Aperture Scintillometer: BLS450, made in Germany) observations was obtained at the A'rou freeze/thaw observation station from Mar. 11 to Jul. 11, 2008. The transmitter (E100°28′16.4″, N38°03′24.3″, 11.2m) and the receiver (E100°27′25.9″, N38°02′18.1″, 11.5m) were 2390m away from each other and the operating altitude was 9.5m. The observation item was the atmospheric refractive index structural parameters  (Cn2). The transmitting frequency was 5HZ and the data were output per minute. The processed data were archived in a 30 minutes cycle. 
     The data were named after WATER_LAS_A'rou_yyyymmdd-yyyymmdd.csv (yyyymmdd-yyyymmdd for observation time). The missing data were marked "None".

2、Keywords

Theme：Surface energy balance,Radiation,Sensible heat flux
Discipline：Atmosphere
Places：Heihe River Basin, the cold region hydrology experimental area in the upper reaches, A'rou flight zone, 
Time：

3、Data details

1.Scale：None

2.Projection：4326

3.Filesize：5.2MB

4.Data format：

4、Space scope

| - | north：38.08 | - |
| --- | --- | --- |
| west：100.41 | - | east：100.55 |
| - | south：38.02 | - |

5、Time frame:2008-03-24 16:00:00+00:00--2012-04-24 10:00:00+00:00

6、Reference method

References to data:

LI Xin, LIU Shaomin, XU Ziwei. WATER: Dataset of LAS (large aperture scintillometer) observations at the A'rou freeze/thaw observation station (2008-2012). A Big Earth Data Platform for Three Poles, doi:10.3972/water973.0161.db2016

References to articles:

Li, X., Li, X.W., Li, Z.Y., Ma, M.G., Wang, J., Xiao, Q., Liu, Q., Che, T., Chen, E.X., Yan, G.J., Hu, Z.Y., Zhang, L.X., Chu, R.Z., Su, P.X., Liu, Q.H., Liu, S.M., Wang, J.D., Niu, Z., Chen, Y., Jin, R., Wang, W.Z., Ran, Y.H., Xin, X.Z., Ren, H.Z. (2009). Watershed Allied Telemetry Experimental Research. Journal of Geophysical Research, 114(D22103), doi:10.1029/2008JD011590.

Liu, S.M., Li, X., Xu, Z.W., Che, T., Xiao, Q., Ma, M.G., Liu, Q.H., Jin, R., Guo, J.W., Wang, L.X., Wang, W.Z., Qi, Y., Li, H.Y., Xu, T.R., Ran, Y.H., Hu, X.L., Shi, S.J., Zhu, Z.L., Tan, J.L., Zhang, Y., & Ren, Z.G. (2018). The Heihe Integrated Observatory Network: A Basin-Scale Land Surface Processes Observatory in China. Vadose Zone Journal, 17(1), 180072. doi:10.2136/vzj2018.04.0072.

7、Supporting project information

The CAS (Chinese Academy of Sciences) Action Plan for West Development Project
National Program on Key Basic Research Project (973 Program

8、Data resource provider

name: XU Ziwei
unit: Beijing Normal University
email: xuzw@bnu.edu.cn

name: LI Xin
unit: 
email: xinli@itpcas.ac.cn

name: LIU Shaomin
unit: Beijing Normal University
email: smliu@bnu.edu.cn