A Big Earth Data Platform for Three Poles

Magnetotelluric data set of Yangbajing Geothermal field, Dangxiong County, Tibet (2021)

1、Description

This data set is the original observation data of magnetotelluric method (MT) collected by the project team in Yangbajing Geothermal field, Dangxiong County, Tibet. The data format is EDI and contains 53 files. The data set contains 4 MT profiles, with the distance between survey lines of about 1km and the distance between survey points of about 500m. The field data acquisition equipment adopts the new SEP ground electromagnetic detection system developed by the Chinese Academy of Sciences. At each MT measuring point, the two horizontal components ex (north-south direction) and ey (east-west direction) of the electric field are measured with a non polarized electrode, and the three components HX (north-south direction), hy (east-west direction) and Hz (plumb bob direction) of the magnetic field are measured with a magnetic sensor. The observation time of each measuring point exceeds 10 hours, and the effective frequency range is 320 hz~0.001 Hz. Through the preprocessing and inversion of the data set, the electrical structure in the depth of 10km in Yangbajing Geothermal field can be obtained, which provides a basis for the location and scale of deep heat sources, heat control and heat conduction structures in the investigation area.

2、Keywords

Theme：Geoheat,thermal regime,Earth Resistivity,Geothermal energy,Tectonics,Magnetotellurics
Discipline：Solid earth
Places：Yangbajing
Time：2021

3、Data details

1.Scale：None

2.Projection：

3.Filesize：2.38MB

4.Data format：None

4、Space scope

| - | north：30.111972222 | - |
| --- | --- | --- |
| west：90.427777778 | - | east：90.519805556 |
| - | south：30.040944444 | - |

5、Time frame:2021-04-30 16:00:00+00:00--2021-08-15 03:59:59+00:00

6、Reference method

References to data:

CHEN   Weiying. Magnetotelluric data set of Yangbajing Geothermal field, Dangxiong County, Tibet (2021). A Big Earth Data Platform for Three Poles, doi:10.11888/SolidEar.tpdc.2724962022

References to articles:

7、Supporting project information

Second Tibetan Plateau Scientific Expedition Program

8、Data resource provider

name: CHEN   Weiying
unit: 
email: cwy@mail.iggcas.ac.cn