A Big Earth Data Platform for Three Poles

WATER: Dataset of ground truth measurements for snow synchronizing with EO-1 Hyperion in the Binggou watershed foci experimental area on Mar. 22, 2008

1、Description

The dataset of ground truth measurements for snow synchronizing with EO-1 Hyperion was obtained in the Binggou watershed foci experimental area on Mar. 22, 2008. Those provide reliable data for retrieval of snow parameters from remote sensing approaches.
     Observation items included: (1) snow surface emissivity by the portable emissivity determinator near the Binggou cold region hydrometerological station; (2) snow density, snow complex permittivity, snow volumetric moisture and snow gravimetric moisture by the snowfork in BG-A from 11:20-13:53 (BJT) on Mar. 2, 2008; (3) snow parameters in BG-A, BG-B, BG-C, BG-D, BG-E and BG-F, and variables including the snow layer temperature by the probe thermometer, the snow grain size by the handheld microscope, snow density by the aluminum case and the snow surface temperature and the snow-soil interface temperature by the handheld infrared thermometer simultaneous with the satellite; (4) the land surface infrared temperature in BG-D, BG-E, BG-B and BG-F during the airborne mission; (5) fresh snow albedo by the total radiometer east to A2; (6) snow spectrum by the portable ASD from Xinjiang Meteorological Administration and Nanjing University, GPS recordings enclosed.
     Two files including raw data and preprocessed data were archived.

2、Keywords

Theme：Albedo,Snow/ice temperature,Snow depth,Snow,Snow particle size,Snow density,Remote Sensing Technology,Snow analyzer,Visible remote sensing,Snow water equivalent
Discipline：Remote Sensing Technology,Cryosphere
Places：Heihe River Basin, the cold region hydrology experimental area in the upper reaches, ice-channel watershed encryption observation area
Time：2008,

3、Data details

1.Scale：None

2.Projection：4326

3.Filesize：914.9MB

4.Data format：EXCEL

4、Space scope

| - | north：38.18839 | - |
| --- | --- | --- |
| west：100.096381 | - | east：100.286566 |
| - | south：38.01113 | - |

5、Time frame:2008-04-03 16:00:00+00:00--2008-04-03 16:00:00+00:00

6、Reference method

References to data:

SHU   Lele, HAN   Xujun, ZHANG   Pu, LIU   Yan, ZHU   Shijie, MA   Hongwei, WANG Xufeng, MA Mingguo, GU  Juan, CHANG   Cun, JIANG   Tenglong, HAO Xiaohua, CAO   Yongpan, XIAO   Pengfeng , DOU   Yan, LI Hua, FANG   Li, BAI   Yunjie, SUN   Jicheng, XU   Zhen, MA   Zhongguo, GE Chunmei, WU Lizong, HUANG  Chunlin, BAI   Yanfen, LIANG   Ji. WATER: Dataset of ground truth measurements for snow synchronizing with EO-1 Hyperion in the Binggou watershed foci experimental area on Mar. 22, 2008. A Big Earth Data Platform for Three Poles, doi:10.3972/water973.0089.db2013

References to articles:

郝晓华, 王建, 车涛, 张璞, 梁继, 李弘毅, 李哲, 白云洁, 白艳芬. 祁连山区冰沟流域积雪分布特征及其属性观测分析. 冰川冻土, 2009, 31(2): 284-292.

7、Supporting project information

The CAS (Chinese Academy of Sciences) Action Plan for West Development Project
National Program on Key Basic Research Project (973 Program

8、Data resource provider

name: GE Chunmei
unit: Cold and Arid Regions Environmental and Engineering Research Institute, Chinese Academy of Sciences
email: gechm@lzb.ac.cn

name: WU Lizong
unit: Cold and Arid Regions Environmental and Engineering Research Institute, Chinese Academy of Sciences
email: wulizong@lzb.ac.cn

name: HAO Xiaohua
unit: Cold and Arid Regions Environmental and Engineering Research Institute, Chinese Academy of Sciences
email: haoxh@lzb.ac.cn

name: WANG Xufeng
unit: Cold and Arid Regions Environmental and Engineering Research Institute, CAS
email: wangxufeng@lzb.ac.cn

name: LI Hua
unit: 
email: 

name: MA Mingguo
unit: Cold and Arid Regions Environmental and Engineering Research Institute, Chinese Academy of Sciences
email: mmg@lzb.ac.cn

name: GU  Juan
unit: 
email: 

name: HUANG  Chunlin
unit: 
email: 

name: CAO   Yongpan
unit: 
email: 

name: ZHANG   Pu
unit: 
email: 

name: LIANG   Ji
unit: 
email: leung@lzb.ac.cn

name: CHANG   Cun
unit: 
email: 

name: MA   Zhongguo
unit: 
email: 

name: BAI   Yanfen
unit: 
email: 

name: ZHU   Shijie
unit: 
email: 

name: SHU   Lele
unit: 
email: 

name: XU   Zhen
unit: 
email: 

name: FANG   Li
unit: 
email: li_fang113@163.com

name: DOU   Yan
unit: 
email: 

name: JIANG   Tenglong
unit: 
email: 

name: MA   Hongwei
unit: 
email: 

name: SUN   Jicheng
unit: 
email: 

name: BAI   Yunjie
unit: 
email: baiyj27@163.com

name: LIU   Yan
unit: 
email: 

name: HAN   Xujun
unit: 
email: 

name: XIAO   Pengfeng 
unit: Nanjing University
email: xiaopf@nju.edu.cn